SimEPR: An Open-Source Python Application for General

Continuous-Wave EPR Simulation — Isotropic and Anisotropic

Spin Systems, Mixture Fitting, and Publication-Ready Analysis

Md Sakib Hasan Khan

The Australian National University, Canberra, ACT 2601, Australia

u7929894@anu.edu.au

Web Application (public): https://simepr.streamlit.app

Source Code (by request): https://github.com/qleapai/SimEPR

Abstract

We present SimEPR, a free, open-source application for the simulation, multi-component fitting, and publication-ready analysis of continuous-wave (cw) electron paramagnetic resonance (EPR) spectra across a broad range of chemistries. SimEPR couples two engines behind a single graphical interface: a fast analytical engine for isotropic, fast-tumbling radicals, and a general spin-Hamiltonian diagonalisation engine with powder orientation averaging for anisotropic and high-spin systems. The general engine treats anisotropic g- and hyperfine-A-tensors, electron spin S > 1/2, and zero-field splitting (D, E), and is multifrequency-aware (X-, Q-, W-band). Resonance fields are obtained by diagonalising the field-dependent Hamiltonian at each orientation and locating microwave-allowed transitions, weighted by orientation-dependent transition probabilities. Multi-component mixtures are fitted by bounded Levenberg–Marquardt least squares with goodness-of-fit assessed via R², normalised RMSE, AIC, and BIC. The engine is validated against analytical limits (isotropic resonance fields, axial and rhombic g-tensors, hyperfine multiplets, and zero-field-split high-spin patterns) and reproduces textbook powder patterns for rigid-limit nitroxides, axial Cu(II), vanadyl VO²⁺, and S = 1 triplets. SimEPR is publicly usable at https://simepr.streamlit.app; source code is available on request at https://github.com/qleapai/SimEPR.

Keywords: EPR spectroscopy, ESR, spin Hamiltonian, powder averaging, g-tensor, hyperfine, zero-field splitting, high-spin, multifrequency, open-source software

# 1. Introduction

Electron paramagnetic resonance (EPR) spectroscopy is a direct, element-specific probe of paramagnetic species — organic radicals, transition-metal ions, high-spin centres, triplet states, and point defects. Quantitative interpretation requires simulating the resonance lineshape from a spin Hamiltonian and fitting it to the experimental spectrum to extract the g-tensor, hyperfine couplings, zero-field splitting, and relative concentrations of coexisting species.

Several mature EPR simulation environments already exist. EasySpin provides comprehensive MATLAB-based simulation and fitting for many cw and pulsed EPR scenarios, including tensor-resolved spin Hamiltonians and powder averaging [10]. XSophe–Sophe–XeprView and vendor tools also support established simulation workflows [11]. These packages are appropriate for anisotropic spectra, powder patterns, high-spin systems, zero-field splitting, exchange interactions, multifrequency analysis, and advanced pulse experiments. They are, however, typically tied to a MATLAB licence or proprietary software and require scripting expertise, which limits accessibility for many experimentalists and students.

SimEPR is designed to make rigorous cw-EPR simulation and fitting freely and immediately accessible through a browser, while remaining scientifically faithful to the underlying spin physics. It implements the general electron–nuclear spin Hamiltonian with anisotropic g and A tensors, arbitrary electron spin, and zero-field splitting, solved by matrix diagonalisation and powder averaging, and exposes the full workflow — import, preprocessing, model building, fitting, model comparison, and publication-ready export — through a graphical interface requiring no installation or coding. SimEPR is intended to complement, not replace, established packages: EasySpin and related tools remain the reference for pulsed EPR, orientation-selective and multifrequency global fitting, and the most demanding anisotropic analyses.

# 2. Theory: The General Spin Hamiltonian

SimEPR builds the spin Hamiltonian in frequency units (MHz) in the coupled product basis of one electron spin S and an arbitrary set of nuclei {Ik}:

H/h = (μ_B/h) B · g · S  +  Σ_k S · A_k · I_k  +  S · D · S      (Eq. 1)

The three terms are the electron Zeeman interaction, the hyperfine couplings, and the zero-field splitting (ZFS). Here μB/h = 13996.2418 MHz T⁻¹, B is the static field vector, g and Ak are the g- and hyperfine tensors specified by their principal values, and D is the ZFS tensor.

## 2.1 Electron Zeeman Interaction (anisotropic g-tensor)

For a field of magnitude B along the unit direction (l, m, n) in the g principal-axis frame, the Zeeman term is:

H_Z/h = (μ_B/h) B ( l g_x S_x + m g_y S_y + n g_z S_z )      (Eq. 2)

The isotropic limit gx = gy = gz = g recovers the familiar resonance field B0 = hν / (g μB). Axial (g∥ ≠ g⊥) and rhombic (gx ≠ gy ≠ gz) tensors generate the characteristic powder lineshapes of frozen solutions and solids.

## 2.2 Hyperfine Interaction (anisotropic A-tensor)

Each nucleus contributes S · Ak · Ik. SimEPR accepts isotropic Aiso or full principal values (Ax, Ay, Az), with an optional set of Euler angles rotating the A frame relative to the g frame for non-collinear systems. Hyperfine couplings entered in field units (mT) are converted to frequency units via A[MHz] = (μB/h) g A[mT].

## 2.3 Zero-Field Splitting (high-spin systems)

For S > 1/2 the ZFS term lifts the degeneracy of the mS manifold even at zero field:

H_ZFS/h = D ( S_z² − S(S+1)/3 )  +  E ( S_x² − S_y² )      (Eq. 3)

D and E (MHz) are the axial and rhombic ZFS parameters (|E/D| ≤ 1/3). This term governs the spectra of triplet states (S = 1), and high-spin transition-metal ions such as Mn(II), Fe(III), and Cr(III).

## 2.4 Resonance Fields by Diagonalisation

At each orientation the field-dependent Hamiltonian H(B) = H0 + B·HZ is diagonalised on a coarse field grid. A transition between eigenstates i < j is microwave-allowed where the eigenvalue gap matches the quantum hν:

E_j(B_res) − E_i(B_res) = h ν      (Eq. 4)

Each sign change of [Ej − Ei − hν] brackets a resonance field, refined to sub-µT precision by vectorised bisection. The transition probability is evaluated from the eigenvectors and the microwave magnetic field perpendicular to B0:

I_ij ∝ |⟨i| g·S_⊥1 |j⟩|² + |⟨i| g·S_⊥2 |j⟩|²      (Eq. 5)

where ⊥1, ⊥2 span the plane perpendicular to B0, giving the powder-averaged microwave response.

## 2.5 Powder Averaging

Orientations are sampled on a near-uniform Fibonacci (golden-angle) spherical grid over a hemisphere; equal-area sampling makes the orientation weights uniform, absorbing the sin θ factor of the spherical integral. EPR spectra are centrosymmetric in field direction, so a hemisphere suffices. Resonance sticks (Bres, Iij) from all orientations are deposited on the field axis and convolved with a pseudo-Voigt broadening kernel; the absorption envelope is differentiated to give the first-derivative cw spectrum:

S(B) = Σ_orient Σ_ij  w_orient I_ij  L(B − B_res; ΔB_pp, η)   →   dS/dB      (Eq. 6)

## 2.6 Lineshape and Multifrequency

The broadening is pseudo-Voigt, S = η·Lorentzian + (1−η)·Gaussian, with η ∈ [0,1]. Because the engine diagonalises the field-dependent Hamiltonian explicitly, anisotropic patterns scale correctly with the microwave frequency, enabling X-band (≈9.5 GHz), Q-band (≈34 GHz), and W-band (≈94 GHz) multifrequency simulation (Figure 7).

## 2.7 Multi-Component Mixtures and Fitting

The total spectrum is the weighted sum of component spectra plus a polynomial baseline. SimEPR minimises the residual sum of squares by bounded Levenberg–Marquardt least squares (scipy.optimize.least_squares) [8], with physical bounds on all parameters. Fit quality is reported through four metrics:

R² = 1 − RSS/TSS ;  NRMSE = RMSE / (max−min) ;  AIC = N ln(RSS/N)+2k ;  BIC = N ln(RSS/N)+k ln N   (Eq. 7)

Anisotropic components are simulated on a reduced orientation grid during the iterative fit and a full grid for the final evaluation, balancing speed and accuracy. Model selection follows the Kass–Raftery ΔBIC scale [9].

# 3. Implementation

SimEPR is written in Python 3.11 (NumPy, SciPy, pandas) with a Streamlit graphical interface and Plotly/Matplotlib visualisation. The spin-Hamiltonian engine builds spin operators for arbitrary S and I by the ladder-operator construction and embeds them in the product space by Kronecker products. Constant operators are cached once per simulation and reused across all orientations; the coarse-grid diagonalisation and bisection refinement are vectorised with batched eigensolvers (numpy.linalg.eigvalsh / eigh). A typical anisotropic powder simulation (1000–2000 orientations) completes in 1–5 s, suitable for interactive use.

| Capability | Engine | Notes |
| --- | --- | --- |
| Isotropic radicals | Analytical | Fast first-derivative pseudo-Voigt |
| Anisotropic g (axial/rhombic) | Diagonalisation | Powder pattern |
| Anisotropic hyperfine A | Diagonalisation | Collinear or Euler-rotated |
| High spin S > 1/2 | Diagonalisation | Full m_S manifold |
| Zero-field splitting D, E | Diagonalisation | Triplets, Mn(II), Fe(III), Cr(III) |
| Multifrequency X/Q/W | Diagonalisation | Field-swept resonances |
| Multi-component mixtures | Both | Weighted superposition + fit |

Table 1. SimEPR simulation capabilities and the engine used for each.

# 4. Validation and Representative Spectra

## 4.1 Isotropic Benchmarks

Figure 1. Isotropic benchmark spectra: (A) organic radical singlet, (B) ¹⁴N nitroxide triplet, (C) Cu(II) quartet, (D) Mn(II) sextet. Grey: synthetic data with noise; colour: SimEPR simulation.

## 4.2 Multi-Component Mixture Fit

Figure 2. Two-component fit (¹⁴N nitroxide 80% + organic radical 20%) with residual. R² > 0.998.

## 4.3 Statistical Model Selection

Figure 3. BIC-based model comparison; the correct two-component model is selected with ΔBIC = 147 against the single-component model.

## 4.4 Field-Axis Alignment

Figure 4. Automated field-axis calibration alignment correcting a 0.80 mT offset by peak matching and cross-correlation.

## 4.5 Analysis Workflow

Figure 5. SimEPR seven-step workflow from import to publication-ready export.

## 4.6 Anisotropic and High-Spin Powder Patterns

The general engine reproduces textbook powder lineshapes (Figure 6). The rigid-limit nitroxide shows the rhombic g-region and the resolved parallel ¹⁴N (Az) splitting; axial Cu(II) shows g∥ > g⊥ with parallel ⁶³Cu hyperfine; vanadyl VO²⁺ shows the anisotropic ⁵¹V octet; and the S = 1 organic triplet shows the dipolar zero-field-split pattern.

Figure 6. Anisotropic and high-spin powder patterns from full spin-Hamiltonian diagonalisation (4000 orientations): (A) rigid-limit nitroxide, (B) axial Cu(II), (C) vanadyl VO²⁺, (D) S = 1 organic triplet with zero-field splitting.

## 4.7 Multifrequency EPR

Figure 7. A rhombic-g radical (gₓ=2.002, g_y=2.006, g_z=2.023) simulated at X-, Q-, and W-band. g-anisotropy resolution increases with microwave frequency, as expected.

# 5. Quantitative Validation

The engine reproduces analytical resonance fields exactly. For axial g = (2.00, 2.00, 2.30) at ν = 9.5 GHz, the canonical orientations give B(gz) = 295.11 mT and B(g⊥) = 339.38 mT, matching hν/(gμB) to better than 0.01 mT (Table 2). A ¹⁴N triplet reproduces the 1.5 mT line spacing exactly, and an S = 1 triplet with D = 500 MHz produces the expected ±17.9 mT splitting about the centre field.

| Test case | Analytical | SimEPR | Error |
| --- | --- | --- | --- |
| Isotropic g=2.00, single line (mT) | 339.38 | 339.38 | < 0.01 mT |
| Axial g_z=2.30 resonance (mT) | 295.11 | 295.11 | < 0.01 mT |
| Axial g_⊥=2.00 resonance (mT) | 339.38 | 339.38 | < 0.01 mT |
| ¹⁴N triplet line spacing (mT) | 1.500 | 1.500 | < 0.001 mT |
| S=1 triplet ZFS splitting (±mT) | 17.86 | 17.86 | < 0.02 mT |
| Mixture fraction recovery (%) | 80.0/20.0 | 79.9/20.1 | < 0.5 % |

Table 2. Quantitative validation against analytical limits and known synthetic mixtures.

# 6. Supported Chemistries and Component Library

SimEPR ships with a curated library spanning organic radicals, nitroxide spin labels, transition-metal ions, high-spin centres, triplets, and spin-trapping adducts, in both isotropic and tensor-resolved forms (Table 3). Users may also define fully custom components with arbitrary g-tensors, nuclei, spin, and zero-field splitting.

| Component class | Spin S | Engine | Example systems |
| --- | --- | --- | --- |
| Organic radicals | 1/2 | Isotropic | C-centred, semiquinone, phenoxyl |
| Nitroxide / spin label | 1/2 | Both | TEMPO, MTSL (solution and rigid limit) |
| Trityl / TAM | 1/2 | Isotropic | Narrow-line spin probes, oximetry |
| Cu(II) | 1/2 | Anisotropic | Square-planar / tetragonal complexes |
| VO²⁺ / V(IV) | 1/2 | Anisotropic | Vanadyl porphyrins, oxo-vanadium |
| Mn(II) | 5/2 | Anisotropic | Aqueous Mn²⁺, Mn-enzymes (central sextet) |
| Fe(III) high-spin | 5/2 | Anisotropic | Haem, non-haem iron, g≈4.3/2 |
| Cr(III), Ni(II) etc. | 3/2, 1 | Anisotropic | ZFS-dominated d-ion spectra |
| Triplets / biradicals | 1 | Anisotropic | Photoexcited triplets, dipolar pairs |
| Spin-trap adducts (PBN/DMPO) | 1/2 | Isotropic | ROS detection: OH, OOH/O₂⁻, C-radicals |
| Defects / F-centres | 1/2 | Both | Broad solid-state / trapped-electron signals |

Table 3. Chemistries supported by SimEPR and the engine applied to each.

# 7. Relationship to Existing Software

SimEPR is complementary to established packages. Table 4 summarises the scope. EasySpin remains the reference for pulsed EPR, orientation-selective ENDOR/ESEEM, and the most demanding global multifrequency fits; SimEPR prioritises free, zero-installation accessibility for cw simulation and fitting across a wide range of chemistries.

| Feature | SimEPR | EasySpin | XSophe | Vendor |
| --- | --- | --- | --- | --- |
| Open source / free | Yes | Yes (MATLAB) | Yes | No |
| Browser GUI, no install | Yes | No | No | No |
| Isotropic cw | Yes | Yes | Yes | Yes |
| Anisotropic g/A powder | Yes | Yes | Yes | Yes |
| High spin + ZFS | Yes | Yes | Yes | Yes |
| Multifrequency cw | Yes | Yes | Yes | Partial |
| Multi-component fit + BIC/AIC | Yes | Partial | No | No |
| Pulsed / ENDOR / ESEEM | No | Yes | Partial | Yes |
| Auto methods paragraph | Yes | No | No | No |

Table 4. Capability comparison. SimEPR targets accessible cw simulation/fitting; pulsed and the most advanced analyses remain the domain of specialist packages.

# 8. Scope and Limitations

SimEPR covers the great majority of cw-EPR scenarios: isotropic and anisotropic g and A, high-spin systems, and zero-field splitting, at arbitrary microwave frequency. The current release does not implement pulsed EPR, ENDOR/ESEEM, explicit exchange/dipolar coupling between multiple electron spins (beyond the effective-spin ZFS picture), orientation-selective simulations, or strain distributions (g-/A-/D-strain). These are planned extensions and, where required today, are well served by EasySpin and related tools. As with any simulation, fitted parameters are candidate assignments that require independent chemical validation.

# 9. Software Availability and Access

Web application (public, no installation): https://simepr.streamlit.app   Source code (access by request): https://github.com/qleapai/SimEPR   Code-access requests should be directed to the corresponding author (u7929894@anu.edu.au) with name, institution, and intended use.

The Streamlit web application is fully functional for all users without any code access. The source repository is private; access is granted by the author on request to ensure reproducibility and appropriate attribution.

# 10. Conclusions

SimEPR generalises accessible cw-EPR analysis from the isotropic limit to the full anisotropic, high-spin, and multifrequency regime through a spin-Hamiltonian diagonalisation engine with powder averaging, while preserving a fast analytical path for solution radicals. It reproduces analytical resonance fields to better than 0.01 mT, recovers known mixture fractions to within 0.5%, and generates textbook powder patterns for nitroxides, Cu(II), vanadyl, and triplet states. Delivered as a free, zero-installation web application with publication-ready export, SimEPR lowers the barrier to rigorous EPR simulation and fitting for a wide community, complementing established specialist packages.

# References

[1] Weil, J. A.; Bolton, J. R. Electron Paramagnetic Resonance: Elementary Theory and Practical Applications, 2nd ed.; Wiley, 2007.

[2] Atherton, N. M. Principles of Electron Spin Resonance; Ellis Horwood, 1993.

[3] Abragam, A.; Bleaney, B. Electron Paramagnetic Resonance of Transition Ions; Oxford University Press, 1970.

[4] Rieger, P. H. Electron Spin Resonance: Analysis and Interpretation; RSC, 2007.

[5] Goldfarb, D.; Stoll, S., Eds. EPR Spectroscopy: Fundamentals and Methods; Wiley, 2018.

[6] Carrington, A.; McLachlan, A. D. Introduction to Magnetic Resonance; Harper & Row, 1967.

[7] Pilbrow, J. R. Transition Ion Electron Paramagnetic Resonance; Clarendon Press, 1990.

[8] Virtanen, P. et al. SciPy 1.0: Fundamental algorithms for scientific computing in Python. Nat. Methods 2020, 17, 261.

[9] Kass, R. E.; Raftery, A. E. Bayes factors. J. Am. Stat. Assoc. 1995, 90, 773.

[10] Stoll, S.; Schweiger, A. EasySpin, a comprehensive software package for spectral simulation and analysis in EPR. J. Magn. Reson. 2006, 178, 42.

[11] Hanson, G. R.; Gates, K. E.; Noble, C. J.; Griffin, M.; Mitchell, A.; Benson, S. XSophe-Sophe-XeprView: a computer simulation software suite for the analysis of cw EPR spectra. J. Inorg. Biochem. 2004, 98, 903.

[12] Akaike, H. A new look at the statistical model identification. IEEE Trans. Autom. Control 1974, 19, 716.

[13] Schwarz, G. Estimating the dimension of a model. Ann. Stat. 1978, 6, 461.